Windows '95

Creating the diskette for Win95 setup

1. Download the file to a temporary directory on your hard drive.

2. Insert a blank, formated disk into the floppy drive.

3. Run the file 701Win95Setup.exe by double-clicking on it from Windows Explorer.

4. Follow the instructions on the screen and it will expand the image onto your diskette drive.

1. Put the driver diskette into the A: diskette drive.

2. From the Control Panel open Add New Hardware.

3. At the Hardware types menu select Sound, video and games controllers.

Click the Have Disk button.

4. From the Select Device Models table, select ESS ES688 AudioDrive.

To install the audio drivers for the TP 701, follow these simple steps:

1. Boot the system to the Windows 95 desktop.

2. Press the Start button on the task bar.

3. Move the pointer to Settings .

4. Left-click on Control Panel .

5. Open the System Properties window by double-clicking on System in Control Panel.

6. Select the Device Manager tab in the System Properties window.

7. Expand the Sound, video and game controllers section in the Device Manager window by left-clicking on the plus sign (+) to the left of the Sound, video and game controllers item in the Device Manager list.

8 Double-click on the ESS ES1688 (or ES688) Plug and Play AudioDrive to bring up the Properties window for the ESS ES1688 Plug and Play AudioDrive.

9. Select the Driver tab in the ESS ES1688 (or ES688) Plug and Play AudioDrive Properties window and left-click on the Change Driver button.

10. Left-click the Have Disk button on the Select Device panel.

11. Insert the Audio Driver diskette into the A: Drive.

12. Left-click the OK button in the Install From Disk window then left-click on the OK button in the Select Device window.

13. Selecting OK or Close in the remaining open windows of the System Properties panel will close those windows and complete installation of the new drivers.

14. Remove the diskette from the A: drive.

15. Make sure that you have selected Use single mode DMA for the Audio Driver. The steps for making this setting are given below:

- Go to Control Panel in Windows 95.

- Double-click Multimedia .

- Click Advanced in the Multimedia Properties window.

- Go to Audio Devices in the list of Multimedia devices .

- Double-click Audio for ESS ES1688 (or ES688) Plug and Play AudioDrive .

- Select Settings .

- Select Use single mode DMA and close all open windows by clicking OK or Close .

16. Restart Windows 95.

Windows 95 Installation instructions - ThinkPad 701

1. Before installing Windows 95:

Before you install Windows 95 on your ThinkPad, you may need to get updated software from IBM or Microsoft.

...

1.2. System program update (Flash BIOS update)

Before you install Windows 95, you are strongly recommended to get the following version of ThinkPad System Program Service Diskette and to update your flash BIOS. Otherwise, Windows 95 may not run as explained in this document. You need to follow the instructions provided by the system program service diskette very carefully to update your flash BIOS program. Your system could be damaged by incorrect operations during this system program update.

Important

You need to update the flash BIOS prior to install Windows 95 on your ThinkPad. In case you have already installed Windows 95 and update the flash BIOS later, you may need to run "Add hardware Wizard" of Windows 95 once again. If some problem still persists, it is recommended to reinstall Windows 95 after the flash BIOS update.

2.0 Install and setup Windows 95 on ThinkPad

Before you proceed

The Windows 95 device drivers for ThinkPad 701C are included in Windows 95. When you install Windows 95 operating system on ThinkPad 701C, the device drivers are installed automatically.

In WIN.INI remove STRTSCR.EXE

Display settings

The video adapter should be configured to "Chips and Tech. Accelerator". The Monitor should be configured to "Standard Display Type: Laptop Display Panel (640X480). The 701cs will support up to 256 colors. These setting can be found under the display Icon in the Windows 95 Control Panel.

Audio Settings

The audio adapter in the Thinkpad 701 is an "ESS ES688 AudioDrive", The latest drivers for this adapter are located on the Thinkpad 701 Audio Features for Windows 95 diskette.

Installing the software

1. Put the driver diskette into the A: diskette drive.

2. From the Control Panel open Add New Hardware.

3. At the Hardware types menu select Sound, video and games controllers. Click the Have Disk button.

4. From the Select Device Models table, select ESS ES688 AudioDrive.

Modem

The Thinkpad 701 modem should be configured as "IBM Thinkpad 701 Internal Modem" The modem supports Baud rates up to 19200.

3.0 PCMCIA Device driver

Windows 95 comes with the integrated PCMCIA support. The PCMCIA controller should be configured as "Cirrus Logic PCIC compatible PCMCIA controller"

4.0 Known limitations with Windows 95:

* Auto suspend not working when docked. If you dock the ThinkPad 701, the auto suspend timer doescause shorter suspend time due to power drain. To turn off PC cards during suspend, do one of the following.

* With ThinkPad 701C System Disk version 3M or later: Press Fn+F11 followed immediately by Fn+Enter to set a flag forcing BIOS to suspend the PCMCIA even if there is a powered card in the slot (with PCMCIA power setting = Auto in Configuration Utility). This should work for modem cards.n't work due to a Adaptec SCSI driver limitation in Windows 95.

* PCMCIA power management limitations: Windows 95 doesn't turn off PC cards before suspend. This would

5.0 System independent information

Suspend command on Start menu

The "Suspend" command should be visible in the "Start" menu above "Shut Down" command. If Windows 95 that you have doesn't enable this "Suspend" command, please take the following steps to correct the problem.

Enable APM functions

1. Click "My Computer"

2. Click "Control Panel"

3. Click "System"

4. Click on "Device Manager" tab

5. Select "System Devices", "Advanced Power Management", "Settings"

6. Verify that "Enable Power management support" is selected

7. Click "OK"

Make suspend command visible

1. Click "My Computer"

2. Click "Control Panel"

3. Click "Power"

4. Select either "Always" or "Only when undocked" (depending on a type of ThinkPad you have) in "Show Suspend command on Start menu" box.

6.0 PCMCIA function limitations

When you inserted a PC card into PCMCIA slots of ThinkPad, the suspend function has the following limitations by design.

* When the system is power by AC, ThinkPad doesn't enter full suspend and enters standby mode.

* When the system is power by battery, ThinkPad always enter suspend mode regardless of the PC card status. You may lose the connection if your PC card is a network card or modem.

The PCMCIA support software in Windows 95 has the following limitations:

* It does not support the ring indicator of PCMCIA modem cards. Therefore, you cannot use ring indications by PCMCIA modems even if it's support by cards.

* It does not support PCMCIA hard disks power down, even if it's supported by hard disks.

* It does not support the system boot from PCMCIA hard disks

The only other glitch that the parallel ports MUST be set to standard parallel or EPP mode, NOT ECP.

FYI - The same instructions work on all current Thinkpads, with different resource settings in a couple of cases. Also the other machines for the most part do not have a problem with D0000 and E0000 so a 64K segment for shared ram is wasteful, but not usually a problem there.